Detailed Plan Design and Development Checklists

The following checklists are provided:

- Plan Design and Development Checklist - General

- Plan Design and Development Checklist  - Traffic Control Design (Signing, Pavement Marking, Guardrail, Lighting, Traffic Signals)

PLAN DESIGN AND DEVELOPMENT CHECKLIST - GENERAL

1.	Review and Verify Reports and Resources

A.	Review environmental document, summation and transcript of public hearing documents, and/or environmental reports (plan sheets should incorporate all necessary mitigation measures and environmental commitments identified)

B.	Review traffic operations report

C.	Review surface thickness and material recommendations

D.	Review linear soil survey recommendations

E.	Review drainage report

F.	Review 90-1 survey and safety review

G.	Gather and review old plans for the proposed project area

H.	Obtain and review survey data

I.	Review milestone for critical activity dates

J.	Schedule preliminary review, PS&E review, and plan completions dates

2.	Coordination

A.	Advise other sections, divisions, districts, city, and/or FHWA of changes made in alignment, drive locations, grades, etc.

- B.	Review environmental considerations with ETS Division – Environmental Services

1.	Army Corp of Engineers Section 404 Permit

a.	Wetland mitigation

b.	Large stream crossings

2.	Army Corp of Engineers Section 408 Permission

3.	Review need for erosion control and develop erosion control plan

4.	Satisfy flood plain requirements

C.	Review coordination of signing, marking, signals, lighting, and guardrail with Design Division – Traffic Section

D.	Review coordination of utility relocations and adjustments with Design Division – Utility Engineering & CADD Support Section, Utility Engineer

1.	Submit preliminary plan and profile sheets and cross section sheets

E.	Review coordination of airport clearance with Design Division – Utility Engineering & CADD Support Section, Utility Engineer

- F.	Review coordination of right of way, easements, encroachments, etc., with ETS Division – Right of Way Services.

G.	Review coordination of bridges and box culverts with Bridge Division

- H.	Review coordination of special provisions (SP) with ETS Division – Technical Services.

- 1.	Review need of common special provisions: http://www.ugpti.org/dotsc/prepguide/index.php

- 2.	Submit required special provisions to ETS Division – Technical Services a minimum of 4 weeks prior to the PS&E

- 3.	Submit required TERO special provisions to ETS Division – Technical Services a minimum of 8 weeks prior to the PS&E

- 4.	SPs to be listed on Table of Contents of plans (include SPs from other sections and divisions)

I.	Review coordination of bench sections and stabilized earth retaining walls with Materials and Research Division

J.	Review coordination of Preliminary Engineering and Cost Maintenance Agreements with the Programming Division or Local Government Division

K.	Review coordination of railroad crossings with Planning/Asset Management Division – Railroad Section

1.	Submit preliminary plan and profile sheets and cross section sheets

2.	Determine if any crossings are to be abandoned or if the number of tracks be reduced

3.	Determine type of crossing: rubber, wood, or asphalt cement (check with city or district)

4.	Determine type of traffic control: signalized, not signalized

5.	Determine clearances

6.	Determine need for pipe crossings, jacked or bored (need standard drawing and railroad request forms for permit)

7.	Include standard drawings for railroad approach and pavement markings

8.	Include appropriate standard note(s) 107-111 to 107-115 (depending on Railroad Company) and pay item to require liability insurance

9.	Check for correct railroad names

L.	City utilities or city projects (participating funds or city funds only)

- NDDOT to complete design

- 2.	City to complete design

- 3.	Include in NDDOT plan sheets, separate plan sheets, tied or not-tied contracts

4.	Review need for city specifications

5.	Review bid items, existing, new, and coordination of similar items

6.	Obtain utility certification or disposition of relocations and adjustments

M.	Maintain project files (document meetings, telephone calls, decisions, etc)

3.	General Plan Development

A.	Develop Geometric Design

1.	Horizontal alignments (tangents, curves, spirals, superelevation)

a.	Check design vehicle

b.	Check truck turning radii

2.	Vertical alignments (grades, crest and sag curves, sight distances, passing zones)

a.	Develop profile grades and PIs (vertical curve lengths calculated from design speed)

b.	Check grade line and topography for hidden intersections or decision sight distance locations

c.	Check grades to fit approaches, driveways, side streets, etc.

d.	Check vertical clearance at bridges

e.	Check grades to fit right of way constraints

3.	Roadway cross section

a.	Review recommended base and pavement structure

b.	Determine profile grade point and cross slopes

c.	Determine lane and shoulder widths

d.	Determine foreslopes in accordance with Design Guidelines

e.	Determine ditch sections and profiles

f.	Determine cut and fill slopes and bench sections (bench sections should be reviewed with Materials and Research)

g.	Determine backslopes

B.	Develop Intersection Design

1.	Develop intersection layout and/or type. Follow applicable Standard Drawings and AASHTO guidance.

a.	Determine design vehicle and check turning radii

b.	Determine channelization/median requirements

c.	Check lane alignments and skew across intersection

d.	Check sight distances

e.	Check need for traffic islands

f.	Check need for left turn lanes

g.	Check need for right turn lanes

h.	Check requirements for tapers and storage lengths

i.	Review location for crosswalks and curb ramps

j.	Review signal locations

k.	Review need for parking restrictions

C.	Develop Earthwork Design

1.	Determine clearing and grubbing

2.	Determine removal of structures

3.	Determine classification of excavation

4.	Determine subcuts

5.	Review borrow and waste

a.	Determine borrow or waste requirements

b.	Determine department optioned borrow and waste when appropriate

c.	Determine mandatory borrow or mandatory waste requirements

(i)	Obtain public interest determination from FHWA for mandatory borrow and excavation sites

- d.	Submit requirements to ETS Division – Right of Way Services

6.	Determine need for slope flattening

7.	Determine shrink and swell factors

8.	Balance earthwork and/or determine borrow needs

9.	Compute haul

10.	Develop mass diagrams

11.	Review need of erosion control and protection

D.	Develop Drainage Design

1.	Determine drainage areas, size pipe, and compute quantities for:

a.	Culverts (centerline and approaches)

b.	Storm drains (pipe, inlets, manholes, castings, etc)

c.	Lift stations

E.	Develop Right of Way Design

1.	Existing permanent right of way

2.	New permanent right of way

3.	Temporary construction easements

4.	Temporary and permanent drainage easements

5.	Access control requirements

6.	Intersection sight distance requirements

7.	Encroachment verification

F.  Prepare environmental, PS&E, and final cost estimates

1.	Traffic Section quantities included

2.	Right of Way Services Section costs included

3.	Bridge Division quantities included

4.	Materials and Research Division quantities included

- 4.	Plan Sheet Development

A.	Section 001 - Title Sheet

__ 	Project numbers (main funding number and secondary funding numbers)

__	PCN number

__ 	Design data (current and forecast traffic, HS bridge live load, pavement design life, etc.)

__ 	Project description (county, project number, location, type of work)

__ 	Governing specifications paragraph

__ 	Length of project (gross and net miles)

__ 	North arrow

__ 	Legal Description (section, township, range)

__ 	Project data/map (begin and end limits, reference points, stations, equations, bridges, exceptions, county lines, interchanges, and highways)

__ 	Plan completion date and signature block

__ 	Mile splits

__	Clearzone distance

__ 	Barrier striping diagram and legend

__ 	Borrow sites and stockpile sites (may show as separate detail sheet)

B.	Section 002 - Table of Contents Sheet

__ 	Section Number, Sheet numbers, and Description

__ 	Sheet numbers included for each contract on multiple contract projects

__ 	List of Standard Drawings (do not number sheets)

__ 	List special provisions (number and description)

C.  Section 004 - Scope of Work Sheet

__ 	Detail showing layout and type of work

__ 	Show borrow and waste locations

__	PE Signature Block

D.	Section 006 - Notes Sheets

__ 	Review list of standard notes and write any additional plan notes

__ 	Check that all incidental items are included in the notes

__ 	Check that all pay items listed in notes are included on the quantity sheets

__ 	Check that all environmental commitments identified in the environmental document and environmental studies are included in the environmental notes or covered by the Standard Specifications.

__	PE Signature Block

E.	Section 008 - Quantities Sheets

__ 	Check that the Item Description is exactly as it is written in the Mainframe or Annual Average Bid Price.

__ 	Check that totals match plan sheet totals

__ 	Check that subtotal match totals

__ 	Show funding splits (participating, non participating, city funds only, alternatives, total)

__ 	Check to include all quantities from other sections and divisions

__ 	Check to include all quantities listed in plan notes and special provisions

__ 	Contract bond, mobilization, railroad insurance, field lab (for tied projects show on the major project only)

F.	Section 010 - Basis of Estimate Sheets

__	Pavement Removal pay items

__ 	Grading pay items:

__	Summary of quantity breakdown (excavation, embankment, etc.)

__	Water quantity for embankment, excavation, aggregate, and dust palliative

__	Topsoil removal (list depth and areas)

__	Additional Sheets

__	Base pay items

__ 	Surfacing pay items

__ 	Pavement marking pay items

__	List core locations and surfacing thickness data (blended base projects)

__	PE Signature Block

- G.	Section 011 - Data Tables, Concrete Pavement Repair Locations, Coring Data, Salvaged Base Summary, Earthwork Summary, Mass Diagram, Borrow Areas, Alternative Pipe Materials Pipe List Table

__ 	Crown rates and superelevation tables

__	Concrete Pavement Repair Locations

__	Coring data and locations

__	Salvage base summary

__	Earthwork

__	Label locations and quantities of borrow entered into mass diagram

__	Label locations and quantities of hauled materials from side roads etc. entered into mass diagram

__	Label average haul

__	Label locations and quantities of special excavation and subcut

__	Label locations and quantities of clearing and grubbing and topsoil removal

__	Label locations and quantities of topsoil borrow

__	Label locations and quantities for approaches, drives, etc.

__	Borrow areas

__	PE Signature Block

H.	Section 020 - General Details Sheets

__ 	Review need of common details

__	Grading

__	Slope flattening

__	Subgrade repair areas

__	Pipe extensions

__	Underdrain areas and associated drawings

__	Flared intersection layouts and approach

__	Bridge end and railroad crossing

__	Erosion control details

__	Milling

__	PCC pavement (dowels, tie bars, joints, reinforcement over pipe, etc.)

__	Ramp layouts

__	Edgedrains and headwalls

__	Edgedrain connection to manhole or inlet

__	Median and turning lane details

__	Cross over and ramp connections

__	Energy dissipater details

__	Lift station details

__	Floating manhole casting

__	Utility, Trench, Pipe backfill details

__	Stabilized Construction Access

__	Wetland easements

__ 	Dimension clarity

__ 	Cross referencing of details by sheet heading

__	PE Signature Block

I.	Section 030 - Typical Sections Sheets (Existing and Proposed)

__ 	Dimensions

__ 	Location by stations and Reference Points (RP)

__ 	Profile and survey location

__ 	Crown rates and superelevation

__ 	Material classifications

__ 	HBP: show aggregate and pavement width, thickness, and areas

__ 	PCC: show aggregate and pavement width, thickness, areas, joints, dowel bars, reinforcing, and tiebars.

__	Curb and gutter

__ 	Edgedrains

__ 	Pipe installations

__	Foreslope, ditch width, and backslope if applicable

__ 	Geotextile Fabric

__ 	Excavation limits and/or areas (common, class, subcut, muck)

__ 	Waste areas (within cross section)

__ 	Sodding/Hydromulching limits

__ 	Existing typical section should show patching with a variable depth (for blended base projects)

__	PE Signature Block

J.	Section 040 - Removals

__	Layout of removal limits

__	PE Signature Block

K.	Section 050 - Inlet and Manhole Summary

__ 	Drainage details

__	Culvert summary sheets (location, type, size, length)

__	Hydraulic data for pipe greater than 30 inches (100 yr and design frequency yr)

__	Storm drain summary sheets (location number, location, description, top elev., invert elev., base elev., riser length, casting type, position of incoming lines and inverts).  This information may be shown on plan and profile for smaller projects.

__	PE Signature Block

L.	Section 051 – Allowable Pipe List

__	Utilize most current template from Plan Preparation Guide Website

__	Show begin and end station/offsets for each pipe

__	Show pipe lengths (and barrel lengths if concrete)

__	List allowable materials as per Design Guidelines

__	Show required diameter for each allowable material

__	If steel pipe is allowed, indicate coating type, corrugations, and minimum thickness

__	Indicated if end sections are flared, traversable, or remove and reset

__	Indicate applicable backfill detail

__	PE Signature Block

M.	Section 060 - Plan & Profile Sheets

__	Dimensional clarity

__	North arrow

__	Label businesses

__	Show and label horizontal alignment control points and ties, curve data and superelevation rates, etc. (move survey and alignment information to layout sheets if sheets are too cluttered)

__	Show and label vertical alignment grades, curve data, sight distance data, locations and stations of barrier stripes, etc.

__	Show and dimension existing right of way, proposed right of way, construction easements, drainage easements, section lines, etc.

__	Show and label driveway widths, lengths, and locations

__	Show and label curb ramps

__	Earthwork

__	Label mile split quantities

__	Label locations and quantities of borrow entered into mass diagram

__	Label locations and quantities of hauled materials from side roads etc. entered into mass diagram

__	Label average haul

__	Label locations and quantities of special excavation and subcut

__	Label locations and quantities of clearing and grubbing and topsoil removal

__	Label locations and quantities of topsoil borrow

__	Label locations and quantities for approaches, drives, etc.

__	Show and label bypass locations, horizontal and vertical alignments, and quantities

__	Show and Label all ditch grades and ditch blocks

__	Show and label retaining walls

__	Culverts

__	Label removal and/or plug locations

__	Storm drains (inlets and manholes)

__	Check number of inlets or manholes to match summary and quantity sheets

__	Check type of inlet and casting

__	Check manhole size to accommodate storm drain line sizes

__	Check elevations

__	Check riser lengths to elevations shown

__	Check offset locations (offset distance to center of riser)

__	Check length of storm drain lines

__	Check if city or district want insulation on inlets

__	Check if city or district want a minimum depth for inlets

__	Water and sanitary lines

__	Check with city on location and installation (may be shown on separate plan and profile sheets)

__	Show and label direction of drainage flow (ditches, culverts, storm drain)

__	Check existing underground utilities for details (elevation and location)

__	Cross reference detail sheets that apply

__	List pay item quantities

__	Make sure all changes are corrected on related sheets and quantities

__	PE Signature Block

N.	Section 070 - Contours Sheets

__	PE Signature Block

O.	Section 075 – Wetlands, Mitigation, and Environmental Sheets

___ Include Wetland and OW Impact Table as the first plan sheet within the section.

If Wetlands are present in Section 75, 76, or 77 include the following:

___ Check that named streams, rivers, sloughs, and lakes are included.  Label name if   identified.

___ Check that the delineated wetlands and other waters are included and line styles used per CADD Standards.

___ Include deep water impact hatching and other water hatching per CADD standards.

___ Include permanent and/or temporary wetland impacts hatching per CADD standards.

___ Provide any onsite mitigation per CADD standards.  Clearly show where and how to construct the mitigation.  Use CADD standards for line styles and hatching, show bottom contour elevation of intended mitigation area (northing and eastings may be needed), include a typical section depicting slopes, elevations, and mitigation limits.  Provide adequate detail within cross sections for site to be built.

___ Show and label wetland numbers, impact acreage and/or linear feet, mitigation site number, acreage, and perimeter contour, grading tie points, north arrow, township section and range, existing and proposed right of way, temporary easements, existing and proposed pipes box culverts and bridges, permanent sediment and erosion control measures that will prohibit vegetative growth (example: riprap), flow lines, avoidance areas, temporary work such as bypasses and cofferdams should also be included.

___ PE Signature Block

P.	Section 076 – Temporary Sediment and Erosion Control

___ Show and label exterior boundary of delineated wetlands and other waters.  Do not hatch temporary or permanent impacts of existing wetlands or other waters per CADD standards.

___ Include all temporary erosion and sediment control measures and hatch per CADD standards.  Provide a legend for clarification.

___ Include quantities for installation and removal of applicable temporary erosion control measures.

___ Include exterior boundary of cultural avoidance areas.  Label areas as “Avoidance Areas” and do not hatch interior boundary.

___ Show and label the following: North arrow, township section and range, existing and proposed right of way and easements, existing pipes box culverts and bridges, existing storm drain inlets, flow lines for the existing conditions within the right of way and areas leaving/entering the right of way, named streams rivers sloughs lakes and label name if identified, roadway alignment and stationing, grading tie points.

___ Do not include proposed drainage structures such as pipes, box culverts, inlets, etc.

___ PE Signature Block

Q.	Section 077 – Permanent Sediment and Erosion Control

___ Show and label final exterior boundary of remaining existing wetlands and proposed wetland mitigation areas per CADD standards.  Do not hatch temporary or permanent impacts of remaining existing wetlands. Do not hatch proposed wetland mitigation areas.

___ Include all permanent erosion and sediment control measures and hatch per CADD standards.  Provide a legend for clarification.

___ Include quantities for installation of permanent erosion control measures.

___ Include exterior boundary of cultural avoidance areas.  Label areas as “Avoidance Areas” and do not hatch interior boundary.

___ Show and label the following: North arrow, township section and range, existing and proposed right of way and easements, remaining and proposed pipes box culverts and bridges, remaining and proposed storm drain inlets, flow lines for the final proposed conditions within the right of way and areas leaving/entering the right of way, named streams rivers sloughs lakes and label name if identified, roadway alignment and stationing, grading tie points.

___ PE Signature Block

R.	Section 080 - Layouts Sheets (Survey Data, Fencing, etc.)

__ 	Survey control point data sheet included in plans

__	If the plan and profile become cluttered, it may be necessary to provide separate sheets for the alignment data, removal data, etc.

__	Dimension clarity

__	PE Signature Block

- S.	Section 090 - Paving Layout Sheets (information may be on plan and profile)

__	Dimension clarity

__	Show survey and office location designation

__	Show and label joint spacing, locations, type, tie bars, seal, etc.

__	Show and label lane and shoulder widths

__	Show and label pavement reinforcement over pipe

__	Show and label driveways, sidewalks, curb ramps, median paving etc.

__	List pay item quantities by page

__	PE Signature Block

T.	Section 100 - Work Zone Traffic Control Sheets

__ 	Develop work zone traffic control note (identify construction phasing, restrictions, maintenance of access, applicable standard drawings, etc.)

__ 	Develop work zone traffic control details and layouts (construction phasing and/or special considerations, show and label signing, devices, markings, etc.)

__ 	Develop work zone accommodations for pedestrians if existing pedestrian activity is present

__ 	Develop “Traffic Control Devices List”

__ 	Review need for special signs

__ 	Review need for traffic control supervisor

__ 	Review work zone speed limits guidelines

__ 	Review edge drop off guidelines and notes

__ 	Review warrants for portable changeable message signs

__ 	Check if detours are required

__ 	Check if temporary bypasses are required

__ 	Give Traffic Section a copy of plans to review the traffic control

__	PE Signature Block

U.	Sections 110 & 120 - Signing and Markings Sheets

__	Advise Design Division   Traffic Section of changes and/or revisions

__	Coordinate data for typical section, horizontal and vertical alignment

__	Obtain estimated quantities, spec and code

__	See Also: Plan Design and Development Checklist   Traffic Control Design in this Appendix

__	PE Signature Block

V.	Section 130 - Guardrail Sheets

__	Advise Design Division   Traffic Section of changes and/or revisions

__	Coordinate data for typical section, horizontal and vertical alignment

__	Obtain estimated quantities, spec and code

__	See Also: Plan Design and Development Checklist   Traffic Control Design in this Appendix

__	PE Signature Block

W.	Sections 140 & 150 - Lighting and Signals Sheets

__	Advise Design Division   Traffic Section of changes and/or revisions

__	Coordinate data for typical section, horizontal and vertical alignment

__	Obtain estimated quantities, spec and code

__	See Also:  Plan Design and Development Checklist   Traffic Control Design in this Appendix

__	PE Signature Block

X.	Section 160 - ITS Sheets

__	PE Signature Box

Y.	Section 170 - Bridge and Box Culverts Sheets

__	Advise Bridge Division of changes and/or revisions

__	Coordinate data for typical section, horizontal and vertical alignment

__	Obtain estimated quantities, spec and code

__	PE Signature Block

Z.	Sections 175 & 180 - Soils Boring Logs & Pit Plats Sheets

__	Insert soil profile and/or cross section sheets form Materials and Research

__	Insert pit plat sheets form Materials and Research

__	Included in plans

AA.  Section 190 - Haul Road Restrictions Sheets

__	Insert haul road restrictions sheets

__	Included in plans

BB.	Section 200 - Cross Sections Sheets

__	If earthwork sheets are not included, add cut and fill quantities to sections

__	Run cross sections with marked points for future reports generated by district

__	Show and label inlets and manholes

__	Show and label approaches

__	Cross reference to plan and profile

__	Check slopes and grade line to match existing terrain and within right of way limits

CC.	Standard Drawings Sheets

__ 	Review list of standard drawings and add appropriate drawings

__ 	Standard Drawings to be listed on Table of Contents of plans (include standards from other Sections and Divisions)

__ 	Include required Standard Drawings in the back of the original plans only. Print the required Standard Drawings from the web at http://www.dot.nd.gov/manuals/design/designmanual/designmanual.htm under Standard Drawings.

DD.	Model Review

___ Verify the proposed model matches the cross sections and also typical sections.  For each typical change randomly check tangents and all curves.  The following items shall be verified.

___ Roadway width

___ Roadway cross slope on tangents and super elevation on curves

___ Graded shoulder elevation and offset in relation to the centerline

___ Cross slope on shoulders

___ Slope of the surfacing treatment slough

___ Pavement elevation and offset in relation to the centerline

___ Roadway foreslope and backslope

___ Ditch bottom width and slope

___ Grades on approaches meet standards

___ Does the transition from one template to another meet the intention of the build.

___ Have all the applicable Supplemental Design Data files been included on the MFT site in accordance to the CADD Manual.

PLAN DESIGN AND DEVELOPMENT CHECKLIST - TRAFFIC CONTROL DESIGN

(SIGNING, PAVEMENT MARKING, GUARDRAIL, LIGHTING, TRAFFIC SIGNALS)

- Signing

  - Check safety review for recommendations for signs with A-frames and other substandard signs.

Comment. _________________________________________________________

  - Compare Sign Locations and Layout information with Sign Summary Sheet.

Comment. _________________________________________________________

  - Check Junction signing for placement and distance from intersections.

Comment. _________________________________________________________

a.	Stop Conditions, Route Turn Markers across the intersections.

Comment. ___________________________________________________

b.	Check lighting plans to see if signs can be placed on light standards.

Comment. ___________________________________________________

c.	Distance and Destination sign legends to be checked with District Maps from Operations Section in Planning and Asset Management Division.

Comment. ___________________________________________________

  - Sign Supports.

a.	Check lengths on summary sheets.

Comment. ___________________________________________________

b.	Check breakaway types - round pipe

1.	Type A Single Post Signs

Comment. ________________________________________________

2.	Type B Two Post Signs with post spacing less than 8 feet.

Comment. ________________________________________________

3.	Type C Two Post or More Signs with post spacing 8 feet or more.

Comment. ________________________________________________

4.	Check fuse joint requirements.

Comment. ________________________________________________

c.	Multi-Direction Breakaway Bases - Round pipe to be used at ramp terminals where post can be struck in any direction.

Comment. ___________________________________________________

d.	Sign Areas: Primary, Secondary, and Interstate Signs

1.	Type IV - Reflective Sheeting. All sign backgrounds except as listed in 2.

Comment. _____________________________________________

2.  Type XI - Reflective Sheeting.  Use for regulatory signs, warning signs (including school signs), and non-lit overhead signs.

Comment. _____________________________________________

  - Check if standards drawings that are required have been provided.

Comment. _________________________________________________________

  - Check if special notes for this project are provided.

Comment. _________________________________________________________

  - Check if Standard Notes are provided.  Section 754 Standard Notes can be found on the web at http://www.ugpti.org/dotsc/prepguide/index.php under Standard Notes.

Comment. _________________________________________________________

  - Check if cost estimate items and cost are correct.

Comment. ________________________________________________________

  - Check sign design layout sheets for station color and area.

Comment. ________________________________________________________

  - Check special assembly layouts for sign sizes and correct station.

Comment. ________________________________________________________

- Pavement Markings

  - Check Pavement Marking Material Selection Chart. The materials shall meet these requirements.

Comment. _________________________________________________________

  - The center line for two-lane, two-way roadways shall be yellow.

Comment. _________________________________________________________

  - The left edge line for divided highways shall be yellow.

Comment. _________________________________________________________

  - Layouts shall have width and color of line shown.

Comment. _________________________________________________________

  - The pay items shall be listed on plan layouts showing the material, color, type of line and quantity. Each width of line shall be totaled. Check totals. Compare line widths and color with what is shown on layouts.

Comment. _________________________________________________________

  - Check if Standard Drawings have been provided.

Comment. _________________________________________________________

  - Check if special notes for this project are provided.

Comment. _________________________________________________________

  - Check if items and costs are provided.

Comment. _________________________________________________________

  - Check if Standard Notes are provided.  Section 762 Standard Notes can be found on the web at http://www.ugpti.org/dotsc/prepguide/index.php under Standard Notes.

Comment. _________________________________________________________

  - Check if special notes are provided.

Comment. _________________________________________________________

C.	Guardrails

  - Check safety reviews to see if guardrail is proposed.

Comment. _________________________________________________________

  - Review environmental document for guardrail requirements.

Comment. _________________________________________________________

  - Determine if guardrail embankment is to be placed with grading or a separate item.

Comment. _________________________________________________________

  - Check to determine if special notes are required.

Comment. _________________________________________________________

  - Check if Standard Drawings have been provided.

Comment. _________________________________________________________

  - Check if items and costs are provided.

Comment. _________________________________________________________

  - Check if standard notes are provided.  Section 764 Standard notes can be found on the web at http://www.ugpti.org/dotsc/prepguide/index.php under Standard Notes.

Comment. _________________________________________________________

D.	Lighting

  - Interim Lighting.

    - Check Layout to determine if permanent lighting will interfere with interim lighting.

Comment. ___________________________________________________

    - Check to determine if pedestrians can negotiate around interim lighting and construction area.

Comment. ___________________________________________________

    - Check to see that underground and overhead utilities do not interfere with placement of poles, anchors, and wires.

Comment. ___________________________________________________

    - Check that permanent signal will fit while interims are operating.

Comment. ___________________________________________________

- e.	Check summary of quantities conduit and cable runs and notes.

1.	Items to be bid as each.

Comment. ________________________________________________

  - Permanent Lighting.

    - Check Layout to determine if permanent lighting will fit interim lighting.

Comment. ___________________________________________________

    - Check to determine if pedestrians will be able to negotiate around light poles.

Comment. ___________________________________________________

    - Check to see if break-away bases are required.

Comment. ___________________________________________________

    - Check location if in the way of construction.

Comment. ___________________________________________________

    - Check to see that underground and overhead utilities do not interfere with placement of poles and mast arms.

Comment. ___________________________________________________

    - Check if festoon circuit is required.

Comment. ___________________________________________________

    - Check that pull box locations are shown on the plan layout.

Comment. ___________________________________________________

    - Check if stations of layout items agree with quantity calculations and quantity sheet.

Comment. ___________________________________________________

    - Check feed point location.

Comment. ___________________________________________________

    - Check summary of quantities conduit and cable runs.

Comment. ___________________________________________________

    - Check if special feed point requirements are needed and a layout.

Comment. ___________________________________________________

    - Check if note is provided when removed equipment is not to become property of the state.

Comment. ___________________________________________________

    - Check if all side street lighting has power provided.

Comment. ___________________________________________________

    - Check if Lighting Standards have been provided.

Comment. ___________________________________________________

    - Check if items and costs are provided.

Comment. ___________________________________________________

  - Check to determine if special notes are required.

Comment. _________________________________________________________

  - Check if standard notes are provided.  Section 770 Standard notes can be found on the web at http://www.ugpti.org/dotsc/prepguide/index.php under Standard Notes.

Comment. _________________________________________________________

  - Check if Utility Company Feed Point Letter is Prepared.

Comment. _________________________________________________________

E.	Traffic Signals

  - Interim Traffic Signal

    - Check Layout to determine if permanent signals are behind interim signals.

Comment. ___________________________________________________

    - Check to determine if pedestrian signals are needed. If replacing in-place signals that are in the way of construction, and pedestrian signals are in place, pedestrian signals are needed in the interim.

Comment. ___________________________________________________

    - Check to see that underground and overhead utilities do not interfere with placement of poles, anchors and placement of span wires.

Comment. ___________________________________________________

    - Check that poles and anchors do not interfere with pedestrians.

Comment. ___________________________________________________

    - Check so that permanent signals will fit while interims are operating.

Comment. ___________________________________________________

    - Check note on coiling enough cable for relocation of signal heads during different phases.

Comment. ___________________________________________________

    - Check if interim item is placed on the plan layout.

Comment. ___________________________________________________

    - Check if interim signal heads have 2 signal heads in each direction and have a dimension of at least 8 feet between.

Comment. ___________________________________________________

    - Check cable runs with signal head numbers.

Comment. ___________________________________________________

    - Check to see if signal head and traffic volume layouts are shown.

Comment. ___________________________________________________

    - Check timing, phasing, cam breakout, notes and flashing requirements.

Comment. ___________________________________________________

    - Check summary of quantities conduit and cable runs and notes.

      - Items to be bid as each.

        - State furnished items.

        - These items to be wood pole mounted.

Comment. ________________________________________________

- m.	Check if interim signals need to be coordinated.

Comment. ___________________________________________________

- Permanent Traffic Signals.

    - Check Layout to determine if permanent signals are behind interim signals.

Comment. ___________________________________________________

    - Check if near side signals are required.

Comment. ___________________________________________________

    - Check if the signal standards are within the clear zone.

Comment. ___________________________________________________

    - Check to determine if pedestrian signals are needed.

Comment. ___________________________________________________

    - Check location so that signal is not in the way of construction and that pedestrian signals are in place near crosswalk.

Comment. ___________________________________________________

    - Check that lane width and all other street dimensions are shown.

Comment. ________________________________________________________

    - Check to see that underground and overhead utilities do not interfere with placement of poles and mast arms.

Comment. ___________________________________________________

    - Check the need for pedestrian actuation push buttons other than on signal poles.

Comment. ___________________________________________________

    - Check if signals have 2 signal heads for through movement in each direction and have a dimension of at least 8 feet between.

Comment. ___________________________________________________

    - Check that signal and pedestrian heads are positioned so they match standard drawings.

Comment. ___________________________________________________

    - Check if pedestrian push button sign location number and direction it=s facing are shown on plan layout.

Comment. ___________________________________________________

    - Check that pull box locations are shown on the plan layout.

Comment. ___________________________________________________

    - Check if emergency vehicle preemption location and number are shown on plans.

Comment. ___________________________________________________

    - Check cable runs with signal head numbers and color coding.

Comment. ___________________________________________________

    - Check that combination signal and light standards have lighting conductors only entering the pole base.

Comment. ___________________________________________________

    - Check to see if signal head and traffic volume layouts are shown.

Comment. ___________________________________________________

    - Check if stations of layout items agree with quantity calculations and quantity sheet.

Comment. ___________________________________________________

    - Check timing, phasing, cam breakout, notes and flashing requirements.

Comment. ___________________________________________________

    - Check if emergency vehicle preemption is used and that phasing is shown.

Comment. ___________________________________________________

    - If existing signals are being removed, show layout and list removed items and quantity.

Comment. ___________________________________________________

    - Check if note is provided when removed equipment is not to become property of the state.

Comment. ___________________________________________________

    - Check traffic signal cross section to be sure the signal location and stationing agree with other layouts. Check if street name signs are the same as shown on signing plans.

Comment. ___________________________________________________

    - Check controller location and direction of door opening.

Comment. ___________________________________________________

    - Check summary of quantities conduit and cable runs and notes.

- 1.	Internal wiring note.

- 2.	Emergency vehicle indicator light conductor.

- 3.	Emergency vehicle detector cable.

- 4.	Indicate internal wire quantities.

- Comment. ________________________________________________

    - Check if signal heads on signal poles at cross section are located correctly.

- Comment. ___________________________________________________

    - Check if signs for signal head requirements are same as signing plan layouts.

Comment. ___________________________________________________

    - Check if signal progression is required in plans and check progression layout for content, etc.

Comment. ___________________________________________________

    - Check if emergency vehicle preemption needs advance detection.

Comment. ___________________________________________________

    - Check to see if interconnect conductor to be placed between intersections. Interconnect shall be in separate conduit. Pull boxes shall be placed every 500 feet maximum.

Comment. ___________________________________________________

- Check to see if railroad signal interconnect is provided or necessary.

Comment. ___________________________________________________

- Detector loops shall be numbered, showing the number of amplifiers, number of turns, sizes, type of loop, number of preformed loops, and number of micro-loops both double and single prop sets.

Comment. ___________________________________________________

- Check if speed monitoring is required.

Comment. ___________________________________________________

- Check if special feed point requirements and layout are needed.

Comment. _________________________________________________________

- Check if Standard Drawings have been provided.

Comment. _________________________________________________________

- Check if items and costs are provided.

Comment. _________________________________________________________

- Check to determine if special notes are required.

Comment. _________________________________________________________

- Check if standard notes are provided.  Section 772 Standard notes can be found on the web at http://www.ugpti.org/dotsc/prepguide/index.php under Standard Notes

Comment. _________________________________________________________

- Check if the utility company feed point letter is prepared.

Comment. _________________________________________________________